LOGIC CIRCUIT DESIGN LAB

VIVA TEST

Class-S4 EC									Time: 40 minutes

- What is the difference between 74 TTL series and 54 TTL series?				(2)

- What does LS stand for in 74 LS 00 IC							(1)

- Which family of digital ICs is having the highest speed? 					(1)

- An AND gate with schematic "bubbles" on its inputs performs the same function as ______ gate.	(1)

- State Demorgan’s theorem									(1)

- Draw the circuit of a controlled inverter.							(1)

- Minimize the function f=ΠM (0,1,2,3,4,7), implement the circuit using AOI logic.	(3)

- Realize a 1 bit Full adder using 2 half adders	.					(2)

- Draw the circuit of a 4 bit ripple carry adder.						(1)

- Compare the propagation delay of a 4 bit ripple carry adder and a 4 bit carry look ahead adder											(2)

- Write the truth table of a 1 bit comparator and implement the circuit using basic gates.(3)

- Which gate produces an output 1 when the inputs are equal?			(1)

- Write the LESSER than expression for a 4 bit comparator				(1)

- Write the truth table of a BCD to Excess 3 code converter.				(3)

- Which code is known as the unit distance code?					(1)

- Write the gray code of 11010101.							(1)

- Mention the differences between sequential logic circuits and combinational logic circuits?										(2)

- Write the excitation table of a D Flip flop and JK Flip flop				(1)

- Explain race around condition? How is it eliminated?				(2)

- Which are the asynchronous inputs of a flip flop? Explain its different conditions.	(2)

- What is meant by modulus of a counter?						(1)

- Differentiate between synchronous and asynchronous counters			(2)